Question 1  A cantilever beam of length 4 meters carries a point load of 10 kN at its free end. What is the maximum bending moment at the fixed support?

A. 20 kN·m

B. 40 kN·m

C. 60 kN·m

D. 80 kN·m

Correct Answer: B — (M = P×L = 10×4 = 40 kN·m)

Question 2  Which of the following is NOT a property of a good aggregate for concrete?

A. Well-graded particle size distribution

B. High absorption capacity

C. Clean and free from organic matter

D. Adequate hardness

Correct Answer: B

Question 3  A soil sample has a water content of 18% and a specific gravity of 2.65. If the void ratio is 0.72, what is the degree of saturation?

A. 56.3%

B. 62.4%

C. 66.3%

D. 72.5%

Correct Answer: C — (S = wGs/e = 0.18×2.65/0.72 = 66.25%)

Question 4  According to NSCP, what is the minimum thickness of a non-prestressed one-way slab not supporting partitions likely to be damaged by large deflections, with one end continuous, if span is 4.5 m?

A. 150 mm

B. 164 mm

C. 180 mm

D. 200 mm

Correct Answer: C — (L/24 = 4500/24 = 187.5, round up → 188 mm, use 180 mm as closest standard)

Question 5  Water flows through a rectangular channel 2.0 m wide at a depth of 0.8 m and a velocity of 1.5 m/s. What is the Froude number?

A. 0.37

B. 0.54

C. 0.76

D. 1.05

Correct Answer: B — (Fr = V/√(g×y) = 1.5/√(9.81×0.8) = 0.536)