Question 1  A simply supported beam of 8 m span carries a concentrated load of 20 kN at 3 m from the left support. What is the reaction at the left support?

A. 8.75 kN

B. 10.00 kN

C. 12.50 kN

D. 15.00 kN

Correct Answer: C — (RL = P×b/L = 20×5/8 = 12.5 kN)

Question 2  In reinforced concrete design, what does the term 'balanced reinforcement ratio' refer to?

A. The ratio at which steel yields simultaneously as concrete reaches its limit strain

B. The ratio at which the beam has equal top and bottom reinforcement

C. The ratio recommended by NSCP for typical slabs

D. The ratio at which shear and moment capacities are equal

Correct Answer: A

Question 3  A soil has a liquid limit of 45% and a plastic limit of 21%. What is the plasticity index?

A. 21%

B. 24%

C. 45%

D. 66%

Correct Answer: B — (PI = LL − PL = 45 − 21 = 24%)

Question 4  Under NSCP, what is the required compressive strength (f'c) used as basis for structural concrete in buildings?

A. 17 MPa

B. 21 MPa

C. 24 MPa

D. 28 MPa

Correct Answer: C

Question 5  A pump delivers water at a flow rate of 0.05 m³/s against a total head of 30 m. If pump efficiency is 75%, what is the required input power?

A. 9.81 kW

B. 14.72 kW

C. 19.62 kW

D. 22.07 kW

Correct Answer: C — (P = γQH/η = 9.81×0.05×30/0.75 = 19.62 kW)