Question 1 A simply supported beam has a span of 6 meters and carries a uniformly distributed load of 12 kN/m. What is the maximum bending moment at midspan?

A. 36 kN·m B. 54 kN·m C. 72 kN·m D. 108 kN·m

Correct Answer: B — (wL²/8 = 12×36/8 = 54 kN·m)

Question 2 Which of the following materials has the highest compressive strength?

A. Structural steel B. Reinforced concrete C. Timber D. Aluminum

Correct Answer: A

Question 3 A soil sample has a void ratio of 0.65 and a specific gravity of 2.70. What is the dry unit weight of the soil? (Unit weight of water = 9.81 kN/m³)

A. 14.20 kN/m³ B. 15.50 kN/m³ C. 16.02 kN/m³ D. 17.80 kN/m³

Correct Answer: C — (γd = Gs×γw / 1+e = 2.70×9.81 / 1.65 = 16.02 kN/m³)

Question 4 According to NSCP, what is the minimum concrete cover for reinforcement bars in a slab not exposed to weather or in contact with the ground?

A. 10 mm B. 20 mm C. 40 mm D. 75 mm

Correct Answer: B

Question 5 A circular pipe has an internal diameter of 300 mm and carries water at a velocity of 2.5 m/s. What is the discharge in the pipe?

A. 0.118 m³/s B. 0.147 m³/s C. 0.176 m³/s D. 0.196 m³/s

Correct Answer: C — (Q = A×V = π/4 × 0.3² × 2.5 = 0.176 m³/s)